# Chapter 1, Solution 20

p30 volt source = 30x(–6) = –180 W

p12 volt element = 12x6 = 72 W

p28 volt e.ement with 2 amps flowing through it = 28x2 = 56 W

p28 volt element with 1 amp flowing through it = 28x1 = 28 W

pthe 5Io dependent source = 5x2x(–3) = –30 W

Since the total power absorbed by all the elements in the circuit must equal zero,

or 0 = –180+72+56+28–30+pinto the element with Vo or

pinto the element with Vo = 180–72–56–28+30 = 54 W

Since pinto the element with Vo = Vox3 = 54 W or Vo = 18 V.