Chapter 1, Problem 27.

A constant current of 3 A for 4 hours is required to

charge an automotive battery. If the terminal voltage

is 10 + t/2 V, where t is in hours,

- how much charge is transported as a result of the charging?

- how much energy is expended?

- how much does the charging cost?  Assume electricity costs 9 cents/kWh.